CNC-2 AXIS  TYPE  CNC  CYLINDRICAL  GRINDER   電腦二軸外圓磨床

| MODEL型式 SPECIFICATION規格 |  |  | CNC ETG-400 ETG-600 | CNC ETG-650 ETG-1000 | CNC ETG-1500 ETG-2000 |
| --- | --- | --- | --- | --- | --- |
| CAPACITY 能量 | DISTANCE BETWEEN CENTERS兩頂心間距離 | m.m. | 400/600 | 650/1000 | 1500/2000 |
|  | MAX.GRINDING DIAMETER 工作最大直徑 | m.m. | φ260(φ300) | φ300(φ400) | φ400 |
|  | # HYDRAULIC TANK-CAPACITY油壓油箱 | L | 10 | 10 | 10 |
|  | COOLANT TANK-CAPACITY 冷卻水箱 | L | 70 | 100 | 100 |
| TABLE 床台 | TRAVERSE SPEED床台速度 | mm/MIN | 0.01-5000 | 0.01-5000 | 0.01-4000 |
|  | TRAVEL PER HANDWHEEL ROTATION手輪進給M.P.G | mm/REV | 0.001,0.01,0.1 | 0.001,0.01,0.1 | 0.001,0.01,0.1 |
|  | SWIVEL ANGLE-CLOCKWISE 旋轉角度工作者邊 | ° | 12/0 | 3/0 | 4/3 |
|  | SWIVELANGLE-COUNTER CLOCKWISE旋轉角度砂輪邊 | ° | 7/12 | 10/9 | 8/6 |
| HEADSTOCK 主軸台 & TAILSTOCK 尾座 | HEADSTOCK WORK CENTER主軸台頂針 | M.T | NO.4 | NO.4 | (NO.4) (NO.5) |
|  | TAILSTOCK WORK CENTER 尾座頂針 | M.T | NO.3 (NO.4) | NO.4 | NO.4 |
|  | WORKHEAD SWIVEL ANGLE-CLOCKWISE 工作者邊 | ° | 30° | 30° | 30° |
|  | WORKHEAD SWIVEL ANGLE-COUNTER CLOCKWISE砂輪邊 | ° | 90° | 90° | 90° |
|  | WORKHEAD SPINDLE SPEED工作主軸轉速 | R.P.M | 0---180 | 0---180 | 0---180 |
| WHEELHEAD 砂輪台 | GRINDING WHEEL SPINDEL SPEED砂輪轉速 | R.P.M | 2085,2495 | 1530,1800 | 1310,1540 |
|  | GRINDING WHEEL SIZE (OD.×WIDTH×ID.) 砂輪直徑 | m.m. | φ400×25-32×φ127 | φ450×38-50×φ127 | φ510×50-75×φ127 |
|  | IN-FEED PER REVOLUTION OF HANDWHEEL (φ) 手輪M.P.G (φ) | m.m/REV | 0.001,0.01,0.1 | 0.001,0.01,0.1 | 0.001,0.01,0.1 |
|  | GRINDING WHEEL SWIVEL 砂輪頭角度 | ° | ±30 | ±30 | ±30 |
|  | INTERNL GRINDING WHEEL SPINDLE SPEED 內孔研磨主軸轉速 | R.P.M | 14000,18000,20000 | 14000,18000,20000 | 14000,18000,20000 |
|  | GRINDING WHEEL SPEED 砂輪台速度 | m.m/MIN | 0.001-3000 | 0.001-3000 | 0.001-3000 |
| ELECTRIC MOTORS 馬達 | WHEEL DRIVE MOTOR 砂輪馬達 | K.W. | 3.7(4P) | 5.5(4P) | 5.5(4P) |
|  | WORK SPINDLE MOTOR 工作台馬達 | K.W. | 0.4 | 0.75 | 1.5 |
|  | HYDRAULIC PUMP MOTOR 油壓馬達 | K.W. | 0.75 | 1.5 | 2.25 |
|  | WHEEL SPINDLE LUBRICATOR MOTOR 砂輪潤滑馬達 | K.W. | 0.2 | 0.2 | 0.4 |
|  | COOLANT PUMP MOTOR 冷卻泵馬達 | K.W. | 0.1 | 0.2 | 0.2 |
|  | SLIDE WAYS LUBRICATOR MOTOR床軌潤滑馬達 | K.W. | 0.1 | 0.1 | 0.1 |
|  | WHEELHEAD DRIVE MOTOR(X-AXIS) 砂輪台馬達 | K.W. | 1.0 | 2.0 | 2.0 |
|  | TABLE DRIVE MOTOR(Z-AXIS)床台馬達 | K.W. | 1.0 | 3.5 | 3.5 |
|  | SURFACE REQUIRED 機械裝置空間 | m.m | 1750×1850/2170×1950 | 2260×4550/3020×4500 | 5050×2250/6050×2250 |
|  | NET WEIGHT機器重量(約) | KGS | 2000/2100 | 3700/4500 | 5200 |

STANDARD ACCESSORIES:					標準附件:

* SWIVEL BASE MOTOR-DRIVEN WORKHEAD		旋轉式主軸台

* LEVER-OPERATED TAILSTOCK				尾座

* WORKHEAD& TAILSTOCK CENTERS			主軸台及尾座頂針

* STANDARD GRINDING WHEEL				標準砂輪

* WHEEL FLANGE AND EXTRACTOR			砂輪法蘭及拆卸器

* WHEEL DRESSER HOLDER					砂輪修整儀

* COOLANT SUPPLY UNIT						冷卻水裝置

* LUBRICANT SUPPLY UNIT					潤滑油裝置

* TABLE SWIVEL INDICATOR					床台旋轉校錶裝置

* SPLASH GUARD							防水板

* DIAMOND DRESSING TOOL					鑽石刀

* TOOLS AND TOOL BOX						手工具

ADDITIONAL EQUIPMENTS:					特殊附件:

* INTERNAL GRINDING ATTACHMENT			內孔研磨裝置

* 3-POINT STEADY REST						三點式穩定扶架

* 3-JAW SELF-CENTERING CHUCK AND PLATE		三爪夾頭及面盤

* MAGNET COOLANT FILTER					磁性冷卻除屑裝置

* WHEEL BALANCING UNIT					砂輪平衡儀

* EXTRA GRINDING WHEEL					備用砂輪

* EXTRA WHEEL FLANGE						備用砂輪法蘭

* EXTRA DIAMOND DRESSING TOOL			備用鑽石刀

* AUTO –diameter check(MARPOSS P1.)			自動直徑量測

* Auto measureing side(MARPOSS T18.)			端面探測裝置

* Ctap and crashcontro(MARPOSS E20.)				防撞裝置

(本公司保有變更規格設計權利不需另行通知)

(The company reserves the right to change specifications and design of its machine without any prior notices.)